Application Note for CIP_Axis_State_Machine AOI errors on power up Rev A 11-12-24

Overview

On power up and first scan of the sample project each instance of the CIP_Axis_State_Machine add on instruction will read the motion group update rate, determine if the virtual axis is rotary or linear, and write the motion group update rate and axis’ unwind value if rotary or 2 billion if linear to the AKD2G drive’s axes AXIS#.EIP.STREAMPERIOD and AXIS#.EIP.UNWIND by virtue of the embedded MSG instructions in the CIP_Axis_State_Machine add on instruction.

Issue

If power is applied to both the PLC and Drives at the same time the drives generally will  take longer to boot and initialize than the PLC. When this occurs the AKD2G drives are not ready for the parameters mentioned above to be written to from the PLC and CIP_Axis_State_Machine AOI and the embedded MSG instructions will fail.

Solution

The following is an example of monitoring the status of the module’s connection (AKD2G) and run the CIP_Axis_State_Machine AOI only if the module (AKD2G) connection status is “Running”.  Keep in mind this is a simple example and there may be more advanced PLC code for monitoring a module. This example does not demonstrate any error handling.

In the CIP Sync sample project navigate to Main Task->MainProgram->MainRoutine_with_CIP_State_Machines routine in the Controller Organizer.

In Rungs 4 and 5 there are CIP_Axis_State_Machine AOIs for Axis 1 and Axis 2 respectively.

In this case there are no module status monitoring/checks in place.

From the Input/Output Instruction Group, add a GSV to rung 4 as shown:

In the GSV instruction field Class Name double-click on the ? field and then click on the down arrow key. Scroll down and select Module and enter.

Next double-click on the ? Instance Name field and then click on the down arrow key. Select the given Module (AKD2G drive) applicable to the related axis (Axis 1 in this case) and press enter. Note this is consistent with the Module’s name under Ethernet in the Controller Organizer as shown below.

Next double-click on the ? Attribute Name field and then click on the down arrow key. Scroll and select in the list “EntryStatus” and press enter.

In the Dest field we will create a tag to store and monitor the status of the module (AKD2G drive). In this example we will use “Axis1_2_Comm_Status” but the naming convention is application dependent and up to the programmer.

Right-click on the ? Dest field and select “New Tag…”.

The “Axis1_2_Comm_Status” tag will hold the value of “EntryStatus” of the Module which is of the data type INT so the tag “Axis1_2_Comm_Status” will also be declared as an INT. First give the tag the name.

Change the Data Type from the default DINT to INT then click OK.

Click Create.

From the Compare Group Instructions add an EQU instruction as shown below.

From the Module Object Help in Studio5000 Help the attribute EntryStatus when it equals 16#4000 or 16384 decimal then EntryStatus=Running. All connections to the module are established and data is transferring.

In the EQU instruction we want to compare the Dest tag from the GSV instruction with the decimal value of 16384. The completed rung is shown below.

Since the Module (AKD2G) in the Sample Project is for Axis 1 and Axis 2, repeat the steps above for Rung 5 with the CIP_Axis_State_Machine add on instruction for Axis 2.