| Title | PLC Demo Program | Device | S300/700 |
| --- | --- | --- | --- |
| Number | 07 |  |  |
| Author | Jochen Karnatz | Firmware | >= 2.25 |
| Date | January 26, 2012 | Function |  |
| Page | 1 of 2 |  |  |

Description

This program is intended to give an impression of the possibilities of macro programming in the S300. It does not represent a real-world application.

A digital input can be used to set a speed, and the current speed is displayed in the terminal program.

Using two user variables, the size of the speed steps can be set, as well as the time between two speed outputs in milliseconds.

A third digital input can be used to switch the program operating mode. The motor moves like the second hand of a clock, 6° per second, and jumps back after one minute. This part of the program demonstrates how counters and timers can be programmed.

Inputs

| Digital Input 1 | In speed mode, each rising edge adds a specific amount to the target speed. |  |
| --- | --- | --- |
| Digital Input 2 | In speed mode, each rising edge subtracts a specific amount from the target speed. |  |
| Digital input 3 | Switches the program operating modes = 0 Speed mode = 1 Clock mode |  |

Outputs

| Digital output 1 | Set to zero in speed mode when the setpoint speed is zero. |
| --- | --- |

User variables

| DPRVAR1 | Time in ms between two outputs of the actual speed |
| --- | --- |
| DPRVAR2 | Speed level set via the digital inputs |

The output can be enabled in the program's speed mode (digital input 3 = 0) by setting the macro variable PRINT = 1. To do this, enter the following in the terminal

M PRINT 1

. With

M PRINT 0

deactivates the output again. Do not be confused by the controller’s output, which may occur in the middle of the input!

Parameter

| IN1MODE 0 | Digital inputs are used by the program |
| --- | --- |
| IN2MODE 0 |  |
| IN3MODE 0 |  |
| IN7MODE 16 | Virtual input 7 is used to start the reference run. |
| IN7TRIG 0 |  |
| O1MODE 23 | Output is used by the program |